Design brief and concept solution: Covering Gailey open drainage channel (Gabilay)

1) Background and problem statement

Figure 1. open canels in Gabilay

Gabiley town has a primary open drainage channel of approximately 700 m length, about 2 m wide and 2 m deep, running through the center of town toward the downstream valley stream. While the channel provides essential stormwater/wastewater conveyance, its open condition has created significant safety and public health risks for nearby communities—especially children, older persons, and persons with disabilities. The municipality has prioritized the channel as a major urban threat due to frequent community complaints and the risk of injuries, accidental falls, and exposure to contaminated runoff.

Figure 2. shows the current condition of cannels in Gabilay

2) Objectives of the intervention

Overall objective: Reduce safety, health, and disaster risk associated with the open drainage channel while maintaining effective drainage performance.

Specific objectives

- Safety: Eliminate fall hazards and reduce injury risks along the 700 m corridor.

- Public health: Reduce exposure to wastewater/stagnant water, odors, and vectors (mosquitoes/flies).

- Drainage performance: Maintain channel capacity for stormwater conveyance and prevent blockage-related flooding.

- Disaster risk management (DRM): Strengthen flood mitigation by ensuring controlled flow, debris tolerance, and access for maintenance.

- Inclusivity: Provide safe, continuous crossings and mobility along the corridor for children and persons with disabilities without creating new barriers.

3) Local conditions and risk context (stormwater, flooding, wastewater).

A multi-dimensional assessment (including environmental disaster risk management) informed the proposed approach. Based on local rainfall and flood behavior, rainy-season flows have historically not overtopped the channel crest, indicating that the channel’s freeboard is generally adequate. However, localized risks remain from:

- sudden runoff peaks during heavy rainfall,

- debris accumulation causing partial blockages,

- unsafe crossings and uncontrolled access points,

- potential wastewater contamination in the open channel.

The design therefore balances risk reduction + hydraulic function + maintainability under local weather exposure patterns.

4) Concept solution selected

Selected concept: TYPE 1b – Combined stainless-steel grated and non-grated drain cover, supported by a lean concrete frame with protected steel slab, installed with a maximum 10 cm above ground level.

Key features

- Low-profile cover (≤10 cm above ground) to avoid creating an “upraised platform,” reduce trip hazards, and support easy movement.

- Hybrid cover configuration: Approximately 1/3 grated/mesh panels + 2/3 solid (non-grated) panels.

- Solid panels improve safety and cleanliness, reduce litter entry, and allow comfortable walking/crossing.

- Grated/mesh sections act as pressure relief and inflow points in case of abnormal runoff or localized overtopping, reducing the likelihood of damage to the cover system.

- Access/maintenance doors: Periodic lockable/secured access points (“doors”) to enable cleaning, inspection, and retrieval without dismantling long sections.

5) Justification for design selection (fit-for-purpose)

This design is selected because it is the most appropriate option for Gabiley’s conditions:

Stormwater and flood resilience:

- The grated sections provide a safe pathway for excess surface water to enter the channel if unusual flooding occurs, lowering stress on the structure and preventing uncontrolled flow across roads and footpaths.

- Local rainfall history indicates overtopping is rare, so the stainless-steel cover is expected to remain durable with limited direct exposure to floodwater impact.

Wastewater and hygiene control: Covering reduces exposure to contaminated water, reduces vector breeding conditions, and improves environmental sanitation along the town center corridor.

Safety and urban functionality: Converting an open hazard into a covered, stable surface significantly reduces fall risks and allows residents to cross the corridor safely at all points (not only at bridges).

Durability and operations: Stainless steel improves resistance to corrosion and wear, while the concrete frame provides structural stability. Maintenance doors ensure the system remains functional over time through routine desilting and debris removal.

6) Inclusivity and safe crossings (children and persons with disabilities)

- The solution incorporates inclusive access by design:

- Barrier-free movement: Keeping the cover height to ≤10 cm reduces obstacles for wheelchairs, crutches, older persons, and caregivers with children.

- Continuous safe crossing: The covered channel provides predictable, stable crossing along the route, reducing reliance on unsafe informal crossings.

- Child safety: Eliminates open-edge fall risk and reduces exposure to contaminated runoff.

- Slip/trip risk reduction: The non-grated sections offer smoother walking surfaces; grated sections should be placed and oriented to minimize snagging and slipping, especially in high-foot-traffic areas.

7) Maintenance, risk management, and sustainability

- Routine cleaning plan: Maintenance doors enable planned desilting and debris clearing before and during rainy seasons.

- Risk of blockage: The hybrid design reduces debris entry (solid panels) while still allowing emergency inflow/pressure relief (grated panels).

- Institutional sustainability: The municipality’s prioritization and involvement supports long-term ownership, monitoring, and enforcement against dumping.

8) Summary (concept in one statement)

Cover the 700 m open drainage channel in central Gabiley using a low-profile (≤10 cm) TYPE 1b combined stainless-steel grated and non-grated cover on a concrete-steel support system, with periodic access doors—delivering a safe, inclusive, and maintainable flood-risk management solution that protects public health while preserving drainage performance.