LOT 1 – Construction and Rehabilitation of Sanitation Facilities

Scope of Work:

•	Construct 50 flood-proof emergency latrines with long-lasting/expandable superstructures, offset pits, vent pipes, lockable doors, and roofing, located at least 30m from any groundwater source and 1.5m above the water table.

•   Construct 8 disability-friendly latrines with ramps, handrails, wider doors,

handwashing stations, and solar lighting for security.

•	Rehabilitate 6 institutional latrines (2 in Waberi ] School, 2 in Howlwadaag School, and 2 in Dinsoor Main Hospital) to restore safe, gender-segregated, and functional facilities.

•	Ensure compliance with WASH Cluster technical standards, protection guidelines, and disability inclusion requirements.

LOT 2 – Rehabilitation of Water Systems

Scope of Work:

•	Rehabilitate 5 shallow wells with installation of new hand pumps, wellhead protection, fencing, and drainage.

•   Rehabilitate 1 strategic borehole (Jilaalow Water Supply System) including:

Rehabilitation of 40m³ elevated water tanks, installation of hybrid solar-powered

pumping system with all electrical and mechanical components, connection to distribution network and water kiosks.

•   Ensure water quality testing and chlorination during commissioning.

LOT 3 – Construction of New Water Points and Pipeline Extension

Scope of Work:

•   Construct 6 new water kiosks in specified IDP locations.

•	Install 3,000 metres of pipeline extension (4-inch PVC main line, 3-inch and 2-inch branches) with fittings, reducers, valves, and associated civil works.

•   Trench excavation (minimum 700mm depth), backfilling, and site restoration.

LOT 4 – Supply and Distribution of Hygiene Kits

Scope of Work:

•	Supply and deliver 800 hygiene kits as per STS specifications (including jerry cans, buckets, soap, aqua tablets, sanitary pads).

•   Distribute kits to designated project locations in coordination with STS staff.

LOT 5 – Supply of Project Vehicle

Scope of Work:

•   Supply a reliable 4x4 project vehicle suitable for field conditions in Somalia.

•	Vehicle must be in excellent mechanical condition (new or very low mileage), with valid documentation.