Construction of Green House

| No | Description | Quantity | Rate | Amount |
| --- | --- | --- | --- | --- |
| 1 | Construction of 9x40 meters Green House in Burao District Under Togdheer Region. (see the package and specification attached below) | 2 pcs |  |  |

Greenhouse Package and Specifications

| Serial Number | Item | Description |
| --- | --- | --- |
| 1 | Construction | Construction of 9 metersx40 meters Green House in two IDPs under Burao District, Togdheer Region. (see the package and specification attached below) |
| 2 | Dimensions | Area: (9mx40m) Peak height: 3.5m Shoulder height: 2.5m |
| 3 | Structure | - Should be pyramidal shape with semi-circular arches inserted into L-shaped columns - Column-to-column distance is a minimum of 3 meters - The structure should be fixed with front and backside X-crosses to protect against wind - The structure should have double front and back side erected straight columns fixed in the arches for structural support - Crop support of at least 10 parallel steel pipes - Inside truss pipes of V-structure connected to crop support and roof arches - 10 parallel lines of segmented profiles fixed on top to hold the cover and separate different layers of covering - Segmented clips set - Clamp sets include peak clamps, U clamps, and pipe clamped welded Us - All steel should be galvanized - Columns diameter = 48mm - Arches diameter = 48mm/42mm |
| 3 | Ventilation | Sideways, front, and back side ventilation with white UV-treated 40 mesh insect net (1.6m width). The height of the mesh will be 1.6 meters high |
| 4 | Cover | - The top roof cover should be made from SOLARIG, extending 1.4 meters on each side. - Ventilation for the sides, front, and back should be made of 40 mesh net. - Flood protection should use SOLARIG material, starting 1 meter above the ground and buried. - The cover should be secured with strong segmented clips and zigzag wire, with the lowest part of the SOLARIG cover buried 0.6 meters into the ground. |
| 5 | Cover Protection | Fish net to prevent wind damage on plastic cover (the plastics should have quality and thinness) |
| 6 | Irrigation | - 700 meters of H/LDPE with a 16mm diameter and a minimum thickness of 0.9mm - 30 meters of 1-inch HDPE pipe - 40 pieces of 16mm rubber - 40 pieces of 16mm connectors - 40 pieces of 16mm mini valves - 40 pieces of 16mm end cups - 3 pieces of 1-inch end cups - 2 pieces of 1-inch female elbows - 3 pieces of 1-inch elbows - 3 pieces of 1-inch stop cocks |
| 7 | Tanks | * 2000 litres plastic tanks perched on a 2.5-meter-high fabricated steel stand with frame and ladder |
| 8 | PPE | Includes suit, goggles, gloves, and mask |
| 9 | Knap Sack Sprayer | 16 litres |
| 10 | Seed trays | 105 holes (15 pcs) |
| 11 | Seed | One season equivalent. Crop type: based on customer requirement |
| 12 | Installation | Construction of the greenhouses, fixing drip irrigation, and tanks + training |
| 13 | Transportation | Burao Districts ( 2 IDPs) – Caaqil-Yare and Barwaaqo IDPs. |